# Problem 1: Arduino-Based Temperature Monitoring System Using DHT22

1. Objective

To develop a temperature monitoring system using an Arduino Uno and a DHT22 sensor that periodically measures ambient temperature in degrees Celsius and displays the readings on the Serial Monitor.

2. Components Required

- Arduino Uno R3

- DHT22 temperature and humidity sensor

- Connecting wires

- Breadboard (optional)

- Tinkercad Circuits or Arduino IDE for programming and testing

3. Sensor Connections and Pin Assignments

DHT22 Sensor

| DHT22 Pin | Arduino Uno Connection |
| --- | --- |
| VCC (+) | 5V |
| DATA (OUT) | Digital pin 2 |
| NC (if present) | Not connected |
| GND (−) | GND |

A pull-up resistor of approximately 4.7 kΩ–10 kΩ between DATA and VCC may be required for a bare DHT22 sensor. Many sensor modules already include this resistor.

The DHT22 is a digital sensor that communicates temperature and humidity measurements through a single data pin. It does not provide an analog voltage output proportional to temperature.

4. Program Logic

- Initialization: Include the DHT sensor library, define the sensor's data pin as digital pin 2, and specify the sensor type as DHT22.

- Serial communication: Initialize serial communication at 9600 baud so that temperature readings can be displayed in the Serial Monitor.

- Sensor initialization: Call dht.begin() to initialize communication with the DHT22 sensor.

- Temperature measurement: Use dht.readTemperature() to retrieve the temperature reading in degrees Celsius.

- Error checking: Use isnan(temperature) to determine whether the reading failed. If it fails, display an error message.

- Output: If the reading is valid, display the temperature followed by the unit °C in the Serial Monitor.

- Periodic measurement: Use delay(2000) to wait two seconds before taking the next reading.

5. Expected Operation

The Serial Monitor displays the measured temperature periodically.

Example output:

DHT22 Temperature Monitor

Temperature: 27.40 C

Temperature: 28.50 C

Temperature: 30.00 C

Temperature: 27.60 C

If the sensor reading fails, the program displays an error message instead of a temperature value.

6. Conclusion

The system uses an Arduino Uno to read digital temperature data from a DHT22 sensor and display the results in degrees Celsius. The readings are updated approximately every two seconds, providing a simple method for monitoring ambient temperature.

The system also includes error checking to identify invalid sensor readings. The DHT22 can measure humidity as well, although this program uses only its temperature measurement function.